[image: image2.png]nebosh





RISK ASSESSMENT
	Task:
	Strip and seal
	Date of Assessment:
	30.11.2009

	RAMS Reference: 
	RAMS019
	Assesor:
	H&S Advisor, Director

	Risk rating:
	2 LOW/UNLIKELY
	Date of last review:
	29.4.2026

	
	
	Version No:
	2


	HAZARD:
	PEOPLE AT RISK:
	RISKS PRIOR TO CONTROLS:
	CONTROL MEASURES:
	RISKS AFTER CONTROLS:

	Trip from trailing wires
	Employee, public, client, contractor
	4 HIGH/LIKELY
	Use of battery operated machine.  Taks completed during low traffic period.  Use of warning signs.  Cone off area prior to completion of task
	2 LOW/UNLIKELY

	Electric shock
	Employee
	4 HIGH/LIKELY
	Use of battery operated machine. Visually check wires for any breaks prior to use.  Keep all wires away from liquid.
	2 LOW/UNLIKELY

	Chemical splash
	Employee
	4 HIGH/LIKELY
	Floor machine must have an anti-splash skirt attached.  Ensure all PPE is work at all times.
	2 LOW/UNLIKELY


METHOD STATEMENT
	Equipment required:
	PPE – gloves & goggles, warning signs, floor machine with anti-splash skirt, drive disc and pad, wet suction machine, stripping tool, abrasive pad, edging tool, bucket and cloths, plastic protective sheet, water based polish, polish applicator and mops, buffing pad, dust control mop


1. Put on personal protective equipment.

2. Assemble equipment, check electrical equipment for safety, particularly plugs and cables.

3. Place warning signs.

4. Insert machine plug once the machine is fully assembled and ready for use and has been visualluy checked.
5. Ventilate if appropriate.  Ensure hands are dry.
6. Prepare stripping solution in a bucket according to manufacturers instructions.

7. Apply stripping solution by mop over an area of about 10 square metres, paying particular attention to the edges.

8. Allow to soak for 5 minuets.

9. Commence stripping  the edges using edging tool, paying particular attention to the corners.

10. Machine scrub, starting furthest from the door, working in am overlapping pattern.

11. Ensure bucket is always behind line of work and machine cable is behind machine and not lying in water.  Do not allow machine cable to become taut.
12. Repeat steps 7-11 until whole area has been scrubbed.

13. After scrubbing place machine on plastic protective sheet.

14. Remove splashes from walls, skirting and furniture as they occur.

15. Remove slurry with wet suction machine, starting on a dry area and moving on to a wet area.

16. Mop whole area with clean water.

17. When floor is clean and polish free, allow to dry.

18. Apply first coat of polish thinly and evenly using the applicator. Start at furthest point from the door. Avoid splashing the skirting.

19. Allow to dry hard.

20. Apply second coat at right angles to first coat.

21. Allow to dry hard.

22. If applicable, buff, then remove any surface duct with dust control mop.

23. On completion, remove plug from power socket and recoil machine cable.

24. Clean equipment and check for safety.

25. Return equipment, materials to store.
26. When floor is completely dry return warning signs to store. When not in use store rotary floor machine handle in upright position.

RISK RATING

Risk ratings are calculated by considering the likelihood of an event occurring along with the severity of the potential impact should an accident occur. After considering existing control measures, values are assigned to the likelihood and severity of the impact from the charts below and these figures multiplied to establish the risk rating. 

[image: image1.png]score ood
1 Negligible Rare
2 Low Unlikely
3 Medium possible
4 High Likely
5 Extreme | Almost Certain





Risk rating meanings 

LOW: The risk is highly unlikely to occur 

MEDIUM: There is an increased likelihood that the risk event will occur at some point 

HIGH: There is a probability that the risk event will occur at some point

EXTREME: There is an expectation that the risk event will occur

71 Main Street Frodsham WA6 7DF
Telephone : 01928 890110  Email: info@cwc360.co.uk
Website: www.cwc360.co.uk
Reg No. 0522 9913  VAT No. 163 7928 75
RAMS100 V2 14.06.2024
[image: image3.jpg]SafeContractor

APPROVED
S,

alcumusgrov®





[image: image4.png]mmmmmmmmmm



[image: image5.png]mmmmmmmmmm



[image: image6.png]mmmmmmmmmm



[image: image7.png]mmmmmmmmmm



[image: image8.png]mmmmmmmmmm



